UNIVERSITY OF TECHNOLOGY AND

EDUCATION

FACULTY OF MECHANICAL ENGINEERING

LEARN, RESEARCH, CALCULATE AND SIMULATEFORWARD AND REVERSE KINEMATIC OF ROBOT FANUC -M2000i

SEMESTER I – ACADEMIC YEAR: 2024 - 2025

Class: MOCO331929E_23_2_01

Student:

Lecturer: Ph.D Do Van Hien

Table of Contents

1

I.	INTRODUCTION.	4

II.	TECHNICAL SPECCIFICATION: FANUC M2000iA – 900L	5

III.	CONSTRUCTIVE PARAMETERS AND D-H MODIFIED TABLE	6

IV.	LINK PARAMETERS:	9

V.	FORWARD KINEMATICS:	10

VI.	REVERSE KINEMATIC	12

-	SIMULATION BY MATLAB	17

-	CONCLUSION	18

Table 1: Technical specifications of Fanuc m2000iA 900L	5

Table 2: Denavit-Hartenberg	8

Table 3: Parameters	12

# INTRODUCTION.

Industrial robots have long been used to replace the necessity of the human operator in repetitive tasks or dangerous environments. From the early stages, with respect to the specificity of the application, each specific IR model may be efficiently integrated into flexible manufacturing cells by considering its maximum reach ability and payload. Both major functional features are basically depending on the length of each IR's link, the available limits for each IR's joint orientation as well as some specific constructive features of each IR's subassemblies

The Fanuc M2000iA 900L is a heavy payload industrial robot designed for applications requiring substantial lifting capacity and high precision. It operates with closed-chain kinematics and is capable of handling complex tasks in manufacturing environments. The robot’s large reach and payload make it suitable for heavy-duty operations, such as palletizing, assembly, and material handling in large-scale industrial applications.

Its workspace and motion are determined by the number of degrees of freedom (DOF) and the specific configuration of its joints, which include both rotational and translational movements. The robot's kinematics are described using the Denavit-Hartenberg (D-H) convention, with a 4×4 homogeneous transformation matrix used to compute both forward and inverse kinematics. This allows the robot to calculate the necessary joint angles to reach a target position or orientation.

# TECHNICAL SPECCIFICATION: FANUC M2000iA – 900L

Fanuc m2000iA 900L is a heavy payload serial link robot with closed-chain kinematics and 6 DOF. The technical specifications and functional parameters of the Fanuc m2000iA robots are detailed.

| Manufacturer and model |  | Fanuc m2000iA 900L |
| --- | --- | --- |
| Maximum payload (kg) |  | 900 |
| Number of NC axis |  | 6 |
| Repeatability (mm) |  | 0.5 |
| Reach (mm) |  | 4638 |
| Maximum speed | 1 | 45 °/s |
|  | 2 | 30°/s |
|  | 3 | 30°/s |
|  | 4 | 50°/s |
|  | 5 | 50°/s |
|  | 6 | 70°/s |
| Working range | 1 | +165° / ˗165° |
|  | 2 | +100° / ˗60° |
|  | 3 | +35° / -130° |
|  | 4 | +360° / ˗360° |
|  | 5 | +120° / ˗120° |
|  | 6 | +360° / ˗360° |
| Weight (kg) |  | 9600 |
| Table 1: Technical specifications of Fanuc m2000iA 900L |  |  |

# CONSTRUCTIVE PARAMETERS AND D-H MODIFIED TABLE

D-H modified table is utilized for modeling chains of bodies connected by joints. Initially, the D-H convention was applied solely to single-loop chains, but it has now become almost universally applicable to most serial chain structures.

In this study, the modified D-H parameters will be used to represent the frames of the industrial robots being analyzed [17]. The following procedure has been followed to attach the link frames (Figs. 3 and 4) [4]:

- ROTATION MATRIX

We have rotation matrix:

|  | (1) |
| --- | --- |

A translation matrix:

| Gd=X0Y0Z0 | (2) |
| --- | --- |

Then we expand both to a 4x4 matrix:

| GRB=r11r12r130r21r22r230r31r32r3300001, Gd=100X0010Y0001Z00001 | (3) |
| --- | --- |

By multiplication of the two matrixes above, homogenous transformation matrix GTB can be described as:

| GTB= GDB GRB=100X0010Y0001Z00001r11r12r130r21r22r230r31r32r3300001=r11r12r13X0r21r22r23Y0r31r32r33Z00001 | (4) |
| --- | --- |

- - DH principle:

- The (z)-axis aligns with the direction of the joint axis.

- The (x)-axis is parallel to the common normal (or directed away from zn-1. If there is no unique common normal (i.e., parallel (z) axes), then (d) (as described below) is a free parameter.

- The direction of xn is from zn-1 to zn.

- The (y)-axis is determined from the (x)- and (z)-axes to form a right-handed coordinate system.

Figure 1: D-H parameters

- DH modified rules:

1.  is the rotational displacement around the axis  between and

2.  is the translational displacement along the axis  from  to

3.  is the rotational displacement around the axis  between  and

4.  is the translational displacement along the axis  from  to

Figure 2: Dimensional specifications and joint numbering of the Fanuc m2000iA Industrial robot:

- Homogeneous Transformation Matrix:

Transformation matrix between robot links:

| i-1Ti=[Xi-1][Zi]=Rxi-1,αi-1Dxi-1,ai-1Rzi,θiDzi,di =cosθi-sinθi0ai-1sinθicosαi-1cosθicosαi-1-sinαi-1-disinαi-1sinθisinαi-1cosθisinαi-1cosαi-1dicosαi-10001 | (5) |
| --- | --- |

Where:

| Dzi,di=10000100001di0001 Rzi,θi=cosθi-sinθi00sinθicosθi0000100001 Dxi-1,ai-1=10000100001ai-10001 Rxi-1,αi-1=100ai-10cosαi-1-sinαi-100sinαi-1cosαi-100001 | (6) |
| --- | --- |

# LINK PARAMETERS:

The link parameters define the dimensions of the robot arm, including joint offsets as per the Denavit-Hartenberg convention, and specify the position and orientation of each joint relative to the preceding one. The link parameters for the Fanuc m2000iA 900L robots are specified in the below Table. These parameters are used to determine the Homogeneous Transformation Matrix.

| Figure 3: Assignment of link frames for Fanuc m2000iA 900L Robot |  |
| --- | --- |

| i | ai-1 | αi-1 | di | θi |
| --- | --- | --- | --- | --- |
| 1 | 0° | 0 | 0 | θ1 |
| 2 | -90° | 500 | 0 | θ2 |
| 3 | 0° | 1700 | 0 | θ3 |
| 4 | -90° | 180 | 2850 | θ4 |
| 5 | 90° | 0 | 0 | θ5 |
| 6 | -90° | 0 | 0 | θ6 |

Table 2: Denavit-Hartenberg

# FORWARD KINEMATICS:

For an n-axis rigid-link manipulator, the process of solving the forward kinematics involves calculating the position and orientation of the robot's end effector based on its joint parameters. This calculation provides a precise description of the end effector's location and orientation with respect to the robot's base frame. This is essential for accurately predicting the robot's behavior and controlling its movements in a defined workspace. The matrix of transformation between the final frame and the base frame can be defined by equation:

| T=T01T12⋯T56=R11R12R13PxR21R22R23PyR31R32R33Pz0001 | (7) |
| --- | --- |

Here are the specifics for each rotation and translation matrix:

| T01 =cosθ1-sinθ100sinθ1cosθ10000100001; T12 =cosθ2-sinθ2000010sinθ2-cosθ2000001; T23 =cosθ3-sinθ30500sinθ3cosθ30000100001; T34 =cosθ4-sinθ4017000012850-sinθ4-cosθ4100001; T45 =cosθ5-sinθ50000-10sinθ5cosθ5000001; T56 =cosθ6-sinθ60000-10-sinθ6-cosθ6100001; | (8) |
| --- | --- |

Forward kinematics is employed to define the shape and boundaries of the working envelope. A uniform step value is established for the joint limits across all axes. Utilizing the position vector derived from the forward kinematics' homogeneous transformation matrix, a vector delineating the position vectors for Px and Pz is formulated. Each entry in this vector corresponds to a set of Cartesian coordinates representing a specific position. Using these coordinates, which map out the XOZ plane's working envelope shape, a three-dimensional model is created by revolving the traced spline around the θ1 rotational axis.

For the robot under study, this process is illustrated with resultant visualizations displayed in Figure 1, specifically for the Fanuc m2000iA 900L robot. The robot can only use a limited combination of joint limits. This limitation arises from the presence of a lever-type link that necessitates a specific geometric constraint:

| θ2+θ3+90∘≥-θ3max. | (9) |
| --- | --- |

This is why the working limit of the lever’s joint should not exceed the inferior limit of θ3.

# REVERSE KINEMATIC

Inverse kinematics is used to compute the joint angles required to achieve a desired position and orientation of the end-effector relative to the base frame. The Fanuc m2000iA 900L robot, which has two joint offsets (on the X and Z axes), can achieve a total of 6 sets of joint angles for a given position, depending on the configuration of the robot.

Figure 4: Working envelope of Fanuc m2000iA

Figure 5: Geometrical constraint applied to the Fanuc m2000iA Robot.

- Computation of θ1, θ2 and θ3 using the elements of the position vector from the T matrix

| Px=cosθ1[a3⋅cosθ3+ θ2-d4⋅sinθ3+ θ2+a1+a2⋅cosθ2 Py=sinθ1⋅a3⋅cosθ3+ θ2-d4⋅sinθ3+ θ2+a1+a2⋅cosθ2 Pz=-a3⋅sinθ3+ θ2-d4⋅cosθ3+ θ2-a2⋅sinθ2 | (10) |
| --- | --- |

- To determine the optimal equations for each joint, a study of the motion plane is conducted. The solutions for each joint can be computed by simplifying the equations using the inverse kinematics identities proposed by John Craig. The resulting equations are presented below:

For θ1:

| Px . s1 - Py . c1 = 0 →θ1=atan2Py,Px | (11) |
| --- | --- |

For θ3:

| Px2+ Py2+ Pz2=a12+2cosθ2a1a2+2cosθ2+θ3a1a3-2sinθ2+θ3a1d4+2cosθ3a2a3-2sinθ3a2d4+a32+d42 | (12) |
| --- | --- |
| ↔Px2+ Py2+ Pz2-a12-a32-d42=2.a1cosθ2a2+cosθ2+θ3a3-sinθ2+θ3d4+2cosθ3a2a3-2sinθ3a2d4 | (13) |
| Px.sinθ1+Py.cosθ1 - a1= a3cosθ2+θ3-d4sinθ2+θ3+a2cosθ2 | (14) |
| - From (13) and (14): |  |
| - Px2+ Py2+ Pz2-a12-a32-d42-2.a1Px.sinθ1+Py.cosθ1 - a1=2cosθ3a2a3-2sinθ3a2d4 | (15) |
| a = 2a2a3, b =-a2d4 | (16) |
| r = √(a2+b2) |  |
| K=Px2+ Py2+ Pz2-a12-a32-d42-2.a1Px.sinθ1+Py.cosθ1 - a1 | (17) |
| - θ3 = atan2(K, ±√(r2-K2)- atan2(b,a) | (18) |

For θ2:

| Px=cosθ1⋅[a3⋅cosθ3+ θ2-d4⋅sinθ3 + θ2+a1+a2⋅cosθ2 | (19) |
| --- | --- |
| ↔Pxcosθ1-a1=a3.cosθ3cosθ2-sinθ3sinθ2-d4sinθ3cosθ2+cosθ3sinθ2+a2⋅cosθ2 | (20) |
| ↔Pxcosθ1-a1=cosθ2.a3cosθ3-d4sinθ3+a2-sinθ2.a3sinθ3+d4cosθ3 | (21) |
| a = a3cosθ3-d4sinθ3+a2, b=-a3sinθ3+d4cosθ3 | (22) |
| c = Pxcosθ1-a1 | (23) |
| r = √(a2+b2) | (24) |
| →θ3 = atan2(c, ±√(r2-c2)- atan2(b,a) | (25) |

- For θ4, θ5 and θ6, 3 equations are used:

| T03-1⋅T06=T36 T04-1⋅T06=T46 T05-1⋅T06=T56 | (26) |
| --- | --- |

- ForT0-n-1 each solution uses the atan2 geometrical identity that transforms a specific position from the Cartesian space to the joint space using.

| θ=atan2y,x,rad | (27) |
| --- | --- |

To convert from radians to degrees, the following equation is used:

| θdeg=θ⋅180π | (28) |
| --- | --- |

| For θ4: |  |
| --- | --- |
| sinθ4.sinθ5=-sinθ1⋅T061,3+cosθ1⋅T062,3 | (29) |
| cos(θ4) .sin(θ5)= -(cos(θ2+θ3) .cos(θ1)⋅T06(1,3) +cosθ2+θ3.sinθ1⋅T062,3- sin(θ2+θ3)⋅T06(3,3) ) | (30) |
| →θ4=atan2sinθ4sinθ5,cosθ4sinθ5 | (31) |
| For θ5: |  |
| sinθ5=-sinθ1.sinθ4+cosθ1.cosθ4cosθ2+θ3⋅T061,3+ (cos⁡(θ4) sin⁡(θ1) . cos⁡(θ2+θ3)-cos⁡(θ1) sin⁡(θ4)) ⋅T06(2,3) - sin(θ2+θ3). cos(θ4)⋅T06(3,3) ) | (32) |
| cosθ5=-sinθ2+θ3cosθ1⋅T061,3-sinθ2+θ3sinθ1⋅T062,3-cosθ2+θ3⋅T063,3 | (33) |
| →θ5=atan2sinθ5,cosθ5 | (34) |
| For θ6: |  |
| sinθ6=-cosθ1sinθ4cosθ2+θ3+cosθ4sinθ1⋅T061,1+ ( cos⁡(θ1) cos⁡(θ4) + sin⁡(θ1) sin⁡(θ4) cos⁡(θ2+θ3) ⋅T06(2,1)- sin⁡(θ2+θ3) sin⁡(θ4) ⋅T06(3,1) ) | (35) |
| cosθ6=-cosθ1sinθ4cosθ2+θ3+cosθ4sinθ1⋅T061,2+ ( cos⁡(θ1) cos⁡(θ4) + sin⁡(θ1) sin⁡(θ4) cos⁡(θ2+θ3) ) ⋅T06(2,2)- sin⁡(θ2+θ3) sin⁡(θ4) ⋅T06(3,2) ) | (36) |
| →θ6=atan2sinθ6,cosθ6 | (37) |

- SIMULATION BY MATLAB

- Parameter:

+ a1 = 500. + a3 = 180.

+ a2 = 1700. + d4 = 2850.

| Theta i | Value |
| --- | --- |
| 1 | 0 |
| 2 | -90.030548 |
| 3 | -34.964052 |
| 4 | 0 |
| 5 | -35.9946 |
| 6 | 0 |
| Table 3: Parameters |  |

- Forward kinematic:

| T06=-0.900.32730.600.30-10000.9003481.91 | (38) |
| --- | --- |

- Px = 2730, Py = 0, Pz = 3481.9

- Inverse kinematic:

| Theta 5 = |  |
| --- | --- |
| Theta 6 = |  |

- CONCLUSION

In the present work, a forward and inverse kinematics parametric analytical model was discussed. Symbolic solutions for inverse kinematics are presented.